Description

M5StickC PLUS is powered by ESP32-PICO-D4 with Bluetooth 4.0 and WiFi and is an upgrade of the original M5StickC with a bigger screen .It is a portable, easy-to-use, open source, IoT development board. This tiny device will enable you to realize your ideas, enrich your creativity, and speed up your IoT prototying. Developing with M5StickC PLUS takes away a lot of the pains from the development process. M5StickC Plus is one of the core devices in M5Stacks product series. The compact body is integrated with rich hardware resources, such as infrared, RTC, Microphone, LED, IMU, Buttons, PMU,etc. Improvements from the regular StickC are a buzzer, bigger screen (1.14-inch, 135 * 240 resolution LCD Screen) and more stable hardware design. This revision increases the display area by 18.7%, and the battery capacity from 95mAh to 120mAh. It also supports the HAT and Unit family of products.

Power switch operation：

Power on ：Press power button for 2 seconds

Power off ：Press power button for 6 seconds

Notice:

Baud rate supported by M5StickC Plus: 1200 ~115200, 250K, 500K, 750K, 1500K

G36/G25 share the same port, when one of the pins is used, the other pin should be set as a floating input

For example, to use the G36 pin as the ADC input, Configuration the G25 pin as FLOATING

setup()

{

M5.begin();

pinMode(36, INPUT);

gpio_pulldown_dis(GPIO_NUM_25);

gpio_pullup_dis(GPIO_NUM_25);

}Copy to clipboardErrorCopied

Product Features

- ESP32-based support BLE 4.2 and WiFi

- Built-in 6-Axis IMU

- Red LED

- IR transmitter

- Microphone

- RTC

- Buttons, LCD(1.14 inch)

- Built-in Lipo Battery

- Extendable Socket

- Built-in Passive Buzzer

- Wearable & Wall mounted

- Development Platform UIFlow, MicroPython, Arduino

Include

- 1x M5StickC Plus

Applications

- Internet of things terminal controller

- Wearable devices

- Stem education product

- DIY creation

Specification

| Resources | Parameter |
| --- | --- |
| ESP32 | 240MHz dual core, 600 DMIPS, 520KB SRAM, Wi-Fi, dual mode Bluetooth |
| Flash Memory | 4MB |
| Power Input | 5V @ 500mA |
| Port | TypeC x 1, GROVE(I2C+I/0+UART) x 1 |
| LCD screen | 1.14 inch, 135*240 Colorful TFT LCD, ST7789v2 |
| Button | Custom button x 2 |
| LED | RED LED |
| MEMS | MPU6886 |
| Buzzer | built-in buzzer |
| IR | Infrared transmission |
| MIC | SPM1423 |
| RTC | BM8563 |
| PMU | AXP192 |
| Battery | 120 mAh @ 3.7V |
| Antenna | 2.4G 3D Antenna |
| PIN port | G0, G25/G36, G26, G32, G33 |
| Operating Temperature | 32°F to 104°F ( 0°C to 40°C ) |
| net weight | 15g |
| Gross weight | 21g |
| Product Size | 48.2*25.5*13.7mm |
| Package Size | 65*25*15mm |
| Case Material | Plastic ( PC ) |

EasyLoader

EasyLoader is a concise and fast program writer, which has a built-in case program related to the product. It can be burned to the main control by simple steps to perform a series of function verification. Please install the corresponding driver according to the device type. M5Core host Please click here to view the CP210X driver installation tutorial, M5StickC/M5StickV/M5StickT/ATOM series can be used without driver)

Schematic

- PDF Download

PinMap

RED LED & IR Transmitter & BUTTON A & BUTTON B

| ESP32 | GPIO10 | GPIO9 | GPIO37 | GPIO39 | GPIO2 |
| --- | --- | --- | --- | --- | --- |
| RED LED | LED Pin |  |  |  |  |
| IR Transmitter |  | Transmitter Pin |  |  |  |
| BUTTON A |  |  | Button Pin |  |  |
| BUTTON B |  |  |  | Button Pin |  |
| Buzzer |  |  |  |  | Buzzer Pin |

TFT LCD

Driver IC：ST7789v2

Resolution：135 * 240

| ESP32 | GPIO15 | GPIO13 | GPIO23 | GPIO18 | GPIO5 |
| --- | --- | --- | --- | --- | --- |
| TFT LCD | TFT_MOSI | TFT_CLK | TFT_DC | TFT_RST | TFT_CS |

GROVE PORT

| ESP32 | GPIO33 | GPIO32 | 5V | GND |
| --- | --- | --- | --- | --- |
| GROVE port | SCL | SDA | 5V | GND |

MIC (SPM1423)

| ESP32 | GPIO0 | GPIO34 |
| --- | --- | --- |
| MICPHONE | CLK | DATA |

6-Axis posture sensor (MPU6886) & power management IC (AXP192)

| ESP32 | GPIO22 | GPIO21 |
| --- | --- | --- |
| 6-Axis IMU sensor | SCL | SDA |
| Power management IC | SCL | SDA |

AXP192

| Microphone | RTC | TFT backlight | TFT IC | ESP32/3.3V MPU6886 | 5V GROVE |
| --- | --- | --- | --- | --- | --- |
| LDOio0 | LDO1 | LDO2 | LDO3 | DC-DC1 | IPSOUT |

Related Link

datasheet

  - ESP32-PICO

  - ST7789v2

  - BM8563

  - MPU6886

  - AXP192

  - SPM1423

Arduino Library

  - M5StickC_PLUS Library

Example

Arduino

- M5StickC Plus facory test code